| EX NO: 7 | CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION |
| --- | --- |
| DATE: |  |

Objectives:

1. To generate a Frequency Modulated (FM) signal in MATLAB using specified carrier and  message signal parameters.

2. To calculate and verify the frequency deviation of the FM signal from the given carrier  frequency and modulating signal parameters.

3. To determine and verify the modulation index of the FM signal using the relationship  between frequency deviation and message frequency.

4. To analyze the effect of message frequency on frequency deviation and modulation  index, and verify their relationship through MATLAB simulation

Procedure

1. Open MATLAB and create a new script file.

2. Clear the command window, workspace variables, and close all previously opened figure  windows using:

Clc;

Clear;

close all;

3. Define the message signal parameters:

● Message amplitude A_m=2V

● Message frequency f_m=500Hz

4. Define the carrier signal parameters:

● Carrier amplitude A_c=1V

● Carrier frequency f_c=10000Hz

5. Define the frequency sensitivity constant k_f, sampling frequency f_s, and simulation  duration T

6. Generate the time vector using:

t = 0:1/fs:T;

7. Generate the message signal using:

m = Am*cos(2*pi*fm*t);

and observe the sinusoidal waveform.

8. Calculate the frequency deviation of the FM signal using:

Δf=kf. Am

9. Calculate the modulation index using:

β=Δf/fm

10. Generate the FM signal using:

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

where the carrier frequency varies according to the message signal.

11. Plot the message signal in the first subplot and label the axes appropriately. 12. Plot the FM signal in the second subplot and observe the variation in instantaneous  frequency.

13. Display the carrier frequency, message frequency, message amplitude, and frequency  sensitivity in the MATLAB command window using fprintf statements.

14. Display the calculated frequency deviation and modulation index values in the command  window.

15. Verify the modulation index by recomputing it using:

β=Δf/fm

and compare the result with the previously calculated value.

16. Use a conditional statement (if-else) to check whether both modulation index values are  equal within a small tolerance.

17. Display the verification result as:

● "Verification Successful" if both values match.

● "Verification Failed" otherwise.

18. Execute the MATLAB program and observe the message signal, FM signal, calculated  frequency deviation, modulation index, and verification result.

PROGRAM (MATLAB CODE)

OBJECTIVE1:

%% EXP 7 - Objective 1: FM Signal Generation

clc;

clear;

close all;

% Given Parameters

Am = 2;              % Message amplitude (V)

fm = 500;            % Message frequency (Hz)

Ac = 1;              % Carrier amplitude (V)

fc = 10000;          % Carrier frequency (Hz)

kf = 250;            % Frequency sensitivity (Hz/V)

fs = 100000;         % Sampling frequency (Hz)

T = 0.01;            % Simulation duration (s)

% Time Vector

t = 0:1/fs:T;

% Modulating and Carrier Signals

m = Am * cos(2 * pi * fm * t);

c = Ac * cos(2 * pi * fc * t);

% FM Modulation

delta_f = kf * Am;

beta = delta_f / fm;

fm_signal = Ac * cos(2 * pi * fc * t + beta * sin(2 * pi * fm * t));

% Plot Time-Domain Waveforms

figure('Name', 'Objective 1: FM Signal Generation', 'NumberTitle', 'off');

subplot(3,1,1);

plot(t, m, 'b', 'LineWidth', 1.5);

title('1. Message Signal m(t)');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

subplot(3,1,2);

plot(t, c, 'k', 'LineWidth', 1);

title('2. Unmodulated Carrier Signal c(t)');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

subplot(3,1,3);

plot(t, fm_signal, 'r', 'LineWidth', 1);

title('3. Frequency Modulated (FM) Signal');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

OBJECTIVE2:
%% EXP 7 - Objective 2: Calculation and Verification of Frequency Deviation

clc;

clear;

close all;

% Parameters

Am = 2;              % Message amplitude (V)

fc = 10000;          % Carrier frequency (Hz)

kf = 250;            % Frequency sensitivity (Hz/V)

% Frequency Deviation Calculation

delta_f = kf * Am;

f_max = fc + delta_f;

f_min = fc - delta_f;

delta_f_measured = (f_max - f_min) / 2;

% Display Verification

fprintf('--- Objective 2: Frequency Deviation Verification ---\n');

fprintf('Frequency Sensitivity (kf)     : %.0f Hz/V\n', kf);

fprintf('Message Amplitude (Am)         : %.0f V\n', Am);

fprintf('Calculated Peak Deviation (Af) : %.2f Hz\n', delta_f);

fprintf('Maximum Instantaneous Freq     : %.2f Hz\n', f_max);

fprintf('Minimum Instantaneous Freq     : %.2f Hz\n', f_min);

fprintf('Verified Peak Deviation        : %.2f Hz\n', delta_f_measured);

if abs(delta_f - delta_f_measured) < 1e-6

fprintf('Status: Frequency Deviation Verification Successful\n');

else

fprintf('Status: Frequency Deviation Verification Failed\n');

end

OBJECTIVE3:
%% EXP 7 - Objective 3: Modulation Index Determination & Verification

clc;

clear;

close all;

% Given Parameters

Am = 2;

fm = 500;

Ac = 1;

fc = 10000;

kf = 250;

% Calculations

delta_f = kf * Am;

beta = delta_f / fm;

beta_verify = delta_f / fm;

% Display Formatted Verification Table

fprintf('--- Objective 3: Modulation Index Verification ---\n');

fprintf('Carrier Frequency (fc)       = %.0f Hz\n', fc);

fprintf('Message Frequency (fm)       = %.0f Hz\n', fm);

fprintf('Message Amplitude (Am)       = %.0f V\n', Am);

fprintf('Frequency Sensitivity (kf)   = %.0f Hz/V\n', kf);

fprintf('Frequency Deviation (Delta f)= %.2f Hz\n', delta_f);

fprintf('Modulation Index (Beta)      = %.2f\n', beta);

fprintf('Verified Modulation Index    = %.2f\n', beta_verify);

% Verification Check

tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance

fprintf('\nVerification Successful\n');

else

fprintf('\nVerification Failed\n');

end

OBJECTIVE4:
-%% EXP 7 - Objective 4: Effect of fm on Frequency Deviation and Beta

clc;

clear;

close all;

% Fixed Modulator Settings

Am = 2;                          % Message amplitude (V)

kf = 250;                        % Frequency sensitivity (Hz/V)

delta_f_const = kf * Am;         % Peak deviation remains constant (500 Hz)

% Test Across Different Message Frequencies

fm_range = [100, 250, 500, 1000, 2000]; % Frequencies in Hz

beta_vals = delta_f_const ./ fm_range;

delta_f_vals = repmat(delta_f_const, size(fm_range));

% Tabular Output to Console

fprintf('--- Objective 4: Invariance of Delta_f and Inverse Scaling of Beta ---\n');

fprintf('  fm (Hz)  |  Delta_f (Hz)  |   Beta (m_f)   \n');

fprintf('--------------------------------------------\n');

for i = 1:length(fm_range)

fprintf('   %4d    |     %6.2f     |     %6.2f     \n', fm_range(i), delta_f_vals(i), beta_vals(i));

end

% Plotting the Relationships

figure('Name', 'Objective 4: Sensitivity to Message Frequency', 'NumberTitle', 'off');

subplot(2,1,1);

plot(fm_range, delta_f_vals, '-s', 'LineWidth', 1.5, 'MarkerFaceColor', 'r');

title('Peak Frequency Deviation (\Delta f) vs. Message Frequency (f_m)');

xlabel('Message Frequency f_m (Hz)'); ylabel('\Delta f (Hz)');

ylim([0, delta_f_const * 1.5]);

grid on;

subplot(2,1,2);

plot(fm_range, beta_vals, '-o', 'LineWidth', 1.5, 'MarkerFaceColor', 'b');

title('Modulation Index (\beta) vs. Message Frequency (f_m)');

xlabel('Message Frequency f_m (Hz)'); ylabel('Modulation Index \beta');

grid on;
OUTPUT:

O1:

O2:
--- Objective 2: Frequency Deviation Verification ---

Frequency Sensitivity (kf)     : 250 Hz/V

Message Amplitude (Am)         : 2 V

Calculated Peak Deviation (Af) : 500.00 Hz

Maximum Instantaneous Freq     : 10500.00 Hz

Minimum Instantaneous Freq     : 9500.00 Hz

Verified Peak Deviation        : 500.00 Hz

Status: Frequency Deviation Verification Successful

O3:
--- Objective 3: Modulation Index Verification ---

Carrier Frequency (fc)       = 10000 Hz

Message Frequency (fm)       = 500 Hz

Message Amplitude (Am)       = 2 V

Frequency Sensitivity (kf)   = 250 Hz/V

Frequency Deviation (Delta f)= 500.00 Hz

Modulation Index (Beta)      = 1.00

Verified Modulation Index    = 1.00

O4:

--- Objective 4: Invariance of Delta_f and Inverse Scaling of Beta ---

fm (Hz)  |  Delta_f (Hz)  |   Beta (m_f)

--------------------------------------------

100    |     500.00     |       5.00

250    |     500.00     |       2.00

500    |     500.00     |       1.00

1000    |     500.00     |       0.50

2000    |     500.00     |       0.25     
RESULT:
1. Generate and visualize carrier, message, and FM signals using MATLAB.

2. Calculate the theoretical frequency deviation and verify it from the simulated FM waveform. 3. Calculate the FM modulation index and verify the relationship β = Δf/fm.

4. Interpret the effect of changing message frequency on modulation index and verify that, for constant  frequency sensitivity and message amplitude, the peak frequency deviation remains constant.