| EXP NO: 2 | CHARACTERISTICS STUDY OF SIDE BAND POWER AND FREQUENCIES IN AM USING MATLAB |
| --- | --- |
| DATE: |  |

Objectives:

i) To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and  modulating signal parameters.

ii) To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of  the AM signal using frequency spectrum analysis in MATLAB.

iii) To calculate the carrier power, sideband power, and total transmitted power of the AM signal  for a given modulation index using MATLAB.

iv) To analyze the effect of varying the modulation index on sideband power distribution and  overall transmission efficiency through MATLAB simulation.

Objective 1:

To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and  modulating signal parameters

PROGRAM (MATLAB CODE)

%% EXP 2 - Objective 1: AM Signal Generation & Demodulation

clc;

clear;

close all;

% Parameters

Am = 1;                 % Modulating signal amplitude (V)

fm = 500;               % Modulating frequency (Hz)

Ac = 5;                 % Carrier amplitude (V)

fc = 5000;              % Carrier frequency (Hz)

fs = 20 * fc;           % Sampling frequency (Hz)

t = 0:1/fs:4/fm;        % Time vector (4 cycles of message)

% Signal Generation

m = Am * sin(2 * pi * fm * t);             % Modulating signal

c = Ac * sin(2 * pi * fc * t);             % Carrier signal

mu = Am / Ac;                              % Modulation index (mu = 0.2)

am = Ac * (1 + mu * sin(2 * pi * fm * t)) .* sin(2 * pi * fc * t); % AM signal

% Envelope Detection

dem = abs(hilbert(am));

dem = dem - mean(dem);                     % Remove DC component

% Plotting

figure('Name', 'Objective 1: AM Generation and Demodulation', 'NumberTitle', 'off');

subplot(4,1,1);

plot(t, m, 'LineWidth', 1.2);

title('1. Modulating / Message Signal');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

subplot(4,1,2);

plot(t, c, 'LineWidth', 1.1);

title('2. Carrier Signal');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

subplot(4,1,3);

plot(t, am, 'LineWidth', 1.1);

title('3. Amplitude Modulated (AM) Signal');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

subplot(4,1,4);

plot(t, dem, 'LineWidth', 1.2);

title('4. Demodulated Message Signal');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

OUTPUT (GRAPH)

Result:

The AM signal was successfully generated and analyzed in MATLAB.

Objective 2:

To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of the  AM signal using frequency spectrum analysis in MATLAB.

PROGRAM:

%% EXP 2 - Objective 2: Sideband Frequencies Verification via FFT

clc;

clear;

close all;

% Parameters

Am = 1;

fm = 500;

Ac = 5;

fc = 5000;

fs = 20 * fc;

t = 0:1/fs:4/fm;

% Modulated Signal

mu = Am / Ac;

am = Ac * (1 + mu * sin(2 * pi * fm * t)) .* sin(2 * pi * fc * t);

% Frequency Spectrum Analysis (FFT)

N = length(am);

f = linspace(-fs/2, fs/2, N);

AM_FFT = abs(fftshift(fft(am))) / N;

% Theoretical Sideband Calculations

USB_theoretical = fc + fm;

LSB_theoretical = fc - fm;

% Display Verification

fprintf('--- Objective 2: Sideband Frequency Verification ---\n');

fprintf('Carrier Frequency (fc) : %d Hz\n', fc);

fprintf('Message Frequency (fm) : %d Hz\n', fm);

fprintf('Theoretical LSB (fc-fm): %d Hz\n', LSB_theoretical);

fprintf('Theoretical USB (fc+fm): %d Hz\n', USB_theoretical);

% Plot Frequency Spectrum

figure('Name', 'Objective 2: AM Frequency Spectrum', 'NumberTitle', 'off');

plot(f, AM_FFT, 'b', 'LineWidth', 1.2);

xlim([fc - 3*fm, fc + 3*fm]);

title('AM Double-Sided Spectrum (Carrier, LSB, USB)');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

OUTPUT GRAPH:

Result:

--- Objective 2: Sideband Frequency Verification ---

Carrier Frequency (fc) : 5000 Hz

Message Frequency (fm) : 500 Hz

Theoretical LSB (fc-fm): 4500 Hz

Theoretical USB (fc+fm): 5500 Hz

Objective 3:

To calculate the carrier power, sideband power, and total transmitted power of the AM signal for  a given modulation index using MATLAB.

PROGRAM:

%% EXP 2 - Objective 3: Power Calculations for a Given Modulation Index

clc;

clear;

close all;

% Parameters

Ac = 5;                  % Carrier amplitude (V)

Am = 1;                  % Message amplitude (V)

R = 1;                   % Normalized load resistance (Ohms)

mu = Am / Ac;            % Modulation index

% Theoretical Power Equations (using peak amplitude across resistance R)

Pc = (Ac^2) / (2 * R);                   % Carrier power

Psb = (mu^2 * Pc) / 4;                   % Power in one sideband (USB or LSB)

Ptsb = 2 * Psb;                          % Total sideband power

Pt = Pc + Ptsb;                          % Total transmitted power

eta = (Ptsb / Pt) * 100;                 % Transmission efficiency (%)

% Display Power Calculations

fprintf('--- Objective 3: Power Distribution Calculations ---\n');

fprintf('Modulation Index (mu)        : %.2f\n', mu);

fprintf('Carrier Power (Pc)           : %.4f W\n', Pc);

fprintf('USB Power                    : %.4f W\n', Psb);

fprintf('LSB Power                    : %.4f W\n', Psb);

fprintf('Total Sideband Power (Ptsb)  : %.4f W\n', Ptsb);

fprintf('Total Transmitted Power (Pt) : %.4f W\n', Pt);

fprintf('Transmission Efficiency (eta): %.2f %%\n', eta);

% Plot Power Distribution Bar Chart

figure('Name', 'Objective 3: Power Distribution', 'NumberTitle', 'off');

bar([Pc, Psb, Psb]);

set(gca, 'XTickLabel', {'Carrier (Pc)', 'USB (P_usb)', 'LSB (P_lsb)'});

ylabel('Power (Watts)');

title(sprintf('AM Power Distribution (\\mu = %.2f)', mu));

grid on;

OUTPUT GRAPH:

Result:

--- Objective 3: Power Distribution Calculations ---

Modulation Index (mu)        : 0.20

Carrier Power (Pc)           : 12.5000 W

USB Power                    : 0.1250 W

LSB Power                    : 0.1250 W

Total Sideband Power (Ptsb)  : 0.2500 W

Total Transmitted Power (Pt) : 12.7500 W

Transmission Efficiency (eta): 1.96 %

OBJECTIVE 4:

To analyze the effect of varying the modulation index on sideband power distribution and  overall transmission efficiency through MATLAB simulation.

PROGRAM:

%% EXP 2 - Objective 4: Sideband Power & Efficiency vs. Modulation Index

clc;

clear;

close all;

% Parameters

Ac = 5;

R = 1;

Pc = (Ac^2) / (2 * R);

mu_values = [0.2, 0.5, 0.8, 1.0];

% Initialize Arrays

P_sb_total = zeros(size(mu_values));

P_total = zeros(size(mu_values));

efficiency = zeros(size(mu_values));

fprintf('--- Objective 4: Variation of mu and Transmission Efficiency ---\n');

fprintf(' mu   |   Pc (W)   |  Ptsb (W)  |   Pt (W)   | Efficiency (%%)\n');

fprintf('------------------------------------------------------------\n');

for i = 1:length(mu_values)

m = mu_values(i);

P_sb_total(i) = (m^2 * Pc) / 2;             % Total sideband power

P_total(i) = Pc + P_sb_total(i);             % Total power

efficiency(i) = (P_sb_total(i) / P_total(i)) * 100; % Efficiency

fprintf('%.2f  |  %8.4f  |  %8.4f  |  %8.4f  |    %6.2f %%\n', ...

m, Pc, P_sb_total(i), P_total(i), efficiency(i));

end

% Plot Efficiency Curve

figure('Name', 'Objective 4: Efficiency vs Modulation Index', 'NumberTitle', 'off');

subplot(2,1,1);

plot(mu_values, efficiency, '-o', 'LineWidth', 1.5, 'MarkerFaceColor', 'b');

title('Transmission Efficiency vs Modulation Index (\mu)');

xlabel('Modulation Index (\mu)');

ylabel('Efficiency (\eta %)');

grid on;

subplot(2,1,2);

plot(mu_values, P_sb_total, '-s', 'LineWidth', 1.5, 'MarkerFaceColor', 'r');

hold on;

plot(mu_values, P_total, '-^', 'LineWidth', 1.5, 'MarkerFaceColor', 'g');

legend('Total Sideband Power (Ptsb)', 'Total Power (Pt)', 'Location', 'northwest');

title('Power Variation with \mu');

xlabel('Modulation Index (\mu)');

ylabel('Power (W)');

grid on;
OUTPUT GRAPH:

RESULT:

--- Objective 4: Variation of mu and Transmission Efficiency ---

mu   |   Pc (W)   |  Ptsb (W)  |   Pt (W)   | Efficiency (%)

------------------------------------------------------------

0.20  |   12.5000  |    0.2500  |   12.7500  |      1.96 %

0.50  |   12.5000  |    1.5625  |   14.0625  |     11.11 %

0.80  |   12.5000  |    4.0000  |   16.5000  |     24.24 %

1.00  |   12.5000  |    6.2500  |   18.7500  |     33.33 %