| EXP. NO: 4 | MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER FOR AM COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING |
| --- | --- |
| DATE: |  |

Objectives

i) To model and simulate the operation of a superheterodyne receiver for an AM communication  system using MATLAB by implementing frequency mixing and intermediate frequency (IF)  signal generation.

ii) To recover the original message signal through intermediate frequency filtering and envelope  detection, and analyze the waveforms of the message, carrier, AM, IF, and recovered signals.

Procedure:

1. Open MATLAB and create a new script.

2. Enter carrier amplitude, carrier frequency, message amplitude, message frequency, sampling  frequency and selected IF.

3. Generate the message signal.

4. Generate the RF carrier signal.

5. Generate the conventional AM signal.

6. Generate the local oscillator; for high-side injection use f_LO = f_RF + f_IF. 7. Multiply the AM/RF signal and local oscillator to perform frequency mixing. 8. Use FFT to identify the mixer sum and difference components.

9. Design an IF band-pass filter centred at f_IF and apply it to the mixer output. 10. Plot the filtered IF waveform and its spectrum.

11. Perform envelope detection on the filtered IF signal.

12. Apply a low-pass filter to recover the message.

13. Plot original and recovered messages together.

14. Compare message, carrier, AM, IF and recovered waveforms.

15. Calculate frequency deviation/error where required and record observations.

PROGRAM (MATLAB CODE) :

OBJECTIVE1:

%% EXP 4 - Objective 1: Frequency Mixing & IF Generation

clc;

clear;

close all;

% Parameters

fs = 100000;          % Sampling frequency (Hz)

t = 0:1/fs:0.02;      % Time vector (20 ms duration)

fm = 500;             % Modulating message frequency (Hz)

fc = 10000;           % RF Carrier frequency (Hz)

fIF = 4550;           % Intermediate Frequency (Hz)

fLO = fc + fIF;       % High-side Local Oscillator (14,550 Hz)

% Signal Generation

m = cos(2 * pi * fm * t);                  % Baseband message signal

c = cos(2 * pi * fc * t);                  % Carrier signal

am = (1 + m) .* cos(2 * pi * fc * t);      % Standard AM signal

% Local Oscillator & Frequency Mixing

LO = cos(2 * pi * fLO * t);                % Local Oscillator signal

mixed = am .* LO;                          % Mixer output: sum & difference frequencies

% IF Filtering

% Isolates difference component (|fLO - fc| = 4550 Hz)

[b, a] = butter(4, 5000 / (fs / 2));

IF = filter(b, a, mixed);

% Plotting Waveforms

figure('Name', 'Objective 1: Superheterodyne Mixing & IF Generation', 'NumberTitle', 'off');

subplot(4,1,1);

plot(t, am, 'LineWidth', 1.1);

title('1. Received AM / RF Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t, LO, 'LineWidth', 1.1);

title(sprintf('2. Local Oscillator Signal (f_{LO} = %d Hz)', fLO));

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t, mixed, 'LineWidth', 1.1);

title('3. Mixer Output (Sum & Difference Components)');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(4,1,4);

plot(t, IF, 'r', 'LineWidth', 1.2);

title(sprintf('4. Filtered Intermediate Frequency (IF) Signal (f_{IF} = %d Hz)', fIF));

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

OBJECTIVE2:

%% EXP 4 - Objective 2: Envelope Detection, Recovery & Comparison

clc;

clear;

close all;

% Parameters

fs = 100000;          % Sampling frequency (Hz)

t = 0:1/fs:0.02;      % Time vector

fm = 500;             % Message frequency (Hz)

fc = 10000;           % RF Carrier frequency (Hz)

fLO = 14550;          % Local Oscillator frequency (Hz)

% Signal Processing Chain

m = cos(2 * pi * fm * t);                  % Transmitted message

c = cos(2 * pi * fc * t);                  % Carrier

am = (1 + m) .* cos(2 * pi * fc * t);      % AM Signal

LO = cos(2 * pi * fLO * t);                % Local Oscillator

mixed = am .* LO;                          % Frequency Mixing

% IF Filter

[b, a] = butter(4, 5000 / (fs / 2));

IF = filter(b, a, mixed);

% Envelope Detection using Hilbert Transform

env = abs(hilbert(IF));

% Display Complete 5-Stage Superheterodyne Receiver

figure('Name', 'Objective 2: Superheterodyne Receiver Waveform Analysis', 'NumberTitle', 'off');

subplot(5,1,1);

plot(t, m, 'LineWidth', 1.2);

title('1. Transmitted Message Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(5,1,2);

plot(t, c, 'LineWidth', 1.1);

title('2. RF Carrier Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(5,1,3);

plot(t, am, 'LineWidth', 1.1);

title('3. Modulated AM Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(5,1,4);

plot(t, IF, 'LineWidth', 1.1);

title('4. Intermediate Frequency (IF) Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(5,1,5);

plot(t, env, 'r', 'LineWidth', 1.2);

title('5. Recovered Message Signal (Envelope Detected)');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

% Quantitative Verification

fprintf('--- Objective 2 Verification ---\n');

fprintf('RF Carrier Frequency (fc) : %d Hz\n', fc);

fprintf('LO Frequency (fLO)        : %d Hz\n', fLO);

fprintf('Standard IF (|fLO - fc|)  : %d Hz\n', abs(fLO - fc));

fprintf('Baseband Message (fm)     : %d Hz\n', fm);

fprintf('Message recovery through IF stage envelope detection completed successfully.\n');

OUTPUT (GRAPH) :

OBJECTIVE1:

OBJECTIVE2:

Result

The superheterodyne receiver was successfully simulated in MATLAB, and the message signal,  carrier signal, AM signal, Intermediate Frequency (IF) signal, and recovered signal were  obtained through frequency mixing and filtering.