| EXP. NO: 6 | GENERATION AND ANALYSIS OF FREQUENCY MODULATED (FM) AND OF PHASE MODULATED (PM) SIGNALS USING MATLAB |
| --- | --- |
| DATE: |  |

Objectives:

To generate and analyze Frequency Modulated (FM) and Phase Modulated (PM) signals using  MATLAB by varying the instantaneous frequency and phase of a carrier signal in accordance  with the message signal.

Procedure

1. Open MATLAB and create a new script file.

2. Define the simulation parameters, including the sampling frequency, time vector, message  frequency, carrier frequency, message amplitude, and carrier amplitude.

3. Generate the message signal using a low-frequency sinusoidal waveform.

4. Generate the carrier signal using a high-frequency sinusoidal waveform.

5. Generate the Frequency Modulated (FM) signal by varying the instantaneous frequency of the  carrier in accordance with the message signal and the selected frequency modulation index.  6. Generate the Phase Modulated (PM) signal by varying the instantaneous phase of the carrier  according to the message signal and the selected phase modulation index.

7. Plot the message signal, carrier signal, FM signal, and PM signal using MATLAB subplots.  8. Observe and compare the time-domain waveforms of the FM and PM signals with the original  message signal.

9. Analyze the variation in the instantaneous frequency of the FM signal and the instantaneous  phase of the PM signal due to changes in the message signal.

10. Compare the characteristics of FM and PM signals by studying their modulation behavior and  waveform variations.

11. Verify the successful generation of FM and PM signals based on the simulation results.

PROGRAM (MATLAB CODE) :
OBJECTIVE1:

%% EXP 6 - Objective 1: FM Signal Generation & Analysis

clc;

clear;

close all;

% Parameters

Am = 1;                 % Modulating signal amplitude (V)

Ac = 1;                 % Carrier amplitude (V)

fm = 5;                 % Message frequency (Hz)

fc = 50;                % Carrier frequency (Hz)

beta = 5;               % Modulation index

fs = 1000;              % Sampling frequency (Hz)

t = 0:1/fs:1;           % Time vector (1 second duration)

% Signals

m = Am * cos(2 * pi * fm * t);                                 % Message signal

c = Ac * cos(2 * pi * fc * t);                                 % Carrier signal

fm_sig = Ac * cos(2 * pi * fc * t + beta * sin(2 * pi * fm * t)); % FM signal

% Frequency Deviation Calculation

delta_f = beta * fm;    % Peak frequency deviation (Hz)

% Display Results

fprintf('--- Objective 1: FM Analysis ---\n');

fprintf('Carrier Frequency (fc)        : %d Hz\n', fc);

fprintf('Message Frequency (fm)        : %d Hz\n', fm);

fprintf('Modulation Index (Beta)       : %d\n', beta);

fprintf('Frequency Deviation (Delta f) : %d Hz\n', delta_f);

% Plotting

figure('Name', 'Objective 1: FM Generation', 'NumberTitle', 'off');

subplot(3,1,1);

plot(t, m, 'LineWidth', 1.2);

title('1. Modulating Message Signal m(t)');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

subplot(3,1,2);

plot(t, c, 'LineWidth', 1.1);

title('2. Carrier Signal c(t)');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;

subplot(3,1,3);

plot(t, fm_sig, 'r', 'LineWidth', 1.1);

title('3. Frequency Modulated (FM) Signal');

xlabel('Time (s)'); ylabel('Amplitude (V)');

grid on;
OBJECTIVE2:

%% EXP 6 - Objective 2: PM Signal Generation & Comparison with FM

clc;

clear;

close all;

% Parameters

Am = 1;

Ac = 1;

fm = 5;

fc = 50;

beta = 5;               % FM modulation index

kp = 5;                 % Phase sensitivity constant (rad/V)

fs = 1000;

t = 0:1/fs:1;

% Signal Generation

m = Am * cos(2 * pi * fm * t);                                 % Message signal

c = Ac * cos(2 * pi * fc * t);                                 % Carrier signal

fm_sig = Ac * cos(2 * pi * fc * t + beta * sin(2 * pi * fm * t)); % FM signal

pm_sig = Ac * cos(2 * pi * fc * t + kp * m);                   % PM signal

% Display

fprintf('--- Objective 2: PM Analysis & Comparison ---\n');

fprintf('Phase Sensitivity (kp)        : %d rad/V\n', kp);

fprintf('Maximum Phase Deviation       : %d radians\n', kp * Am);

fprintf('Comparison: FM instantaneous phase integrates m(t), while PM tracks m(t) directly.\n');

% Plotting Complete Comparison

figure('Name', 'Objective 2: FM vs PM Comparison', 'NumberTitle', 'off');

subplot(4,1,1);

plot(t, m, 'LineWidth', 1.2);

title('1. Modulating Message Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t, c, 'LineWidth', 1.1);

title('2. Carrier Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t, fm_sig, 'b', 'LineWidth', 1.1);

title('3. Frequency Modulated (FM) Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(4,1,4);

plot(t, pm_sig, 'm', 'LineWidth', 1.1);

title('4. Phase Modulated (PM) Signal');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

OUTPUT :

O1:

O2: 
RESULT:
The Frequency Modulated (FM) and Phase Modulated (PM) signals were successfully  generated using MATLAB by varying the instantaneous frequency and phase of the carrier  signal according to the message signal.

• Frequency Deviation (Δf) = 25 Hz

• FM Modulation Index (β) = 5